Faculty of Science, Engineering and Technology

# Computer Systems

Week 8

Part 8.1: Matchsticks

Part 8.2: Matchsticks

Question 8.2.2(a): What is the condition that needs to be satisfied in order for this loop to occur?  Write this as a comparison using an inequality (ie., less than, greater than, less than or equal, greater than or equal)

The condition that needs to be satisfied for this loop to occur is the input must be from 1 to 3, and not smaller than 0 and greater than 3.

Question 8.2.2(b): What two ARM assembly instructions could be used to create a branch that only occurs under this condition?

Two ARM assembly instructions could be used to create a branch that only occurs under this condition is BLT (Branch Less Than) and BGT (Branch Greater Than)

Question 8.2.2(c): Based on the instructions you outlined in 8.2.2(b), what status bit would be set to 1 if the loop was to repeat?

If the loop is to repeat, it means that the comparison did not find the two values to be equal. Therefore, the Z (Zero) status bit would not be set to 1 (i.e., Z = 0). In this case, the loop continues until the values are equal, at which point the Z bit will be set to 1, causing the program to exit the loop

Question 8.2.2(d): What are all the modifications needed to the current program to implement this feature? Make the required modifications to your program to perform the task.

The modifications needed to the current program to implement this feature is add:

+ “CMP R3,#0” which means that compare the value of R0 after “SUB R0,R0,R3” to 0.

+ “BEQ stop” indicates that branch equal to 0 stop the program.

Part 8.3: Implementing computer player

Question 8.3.1(a): What bit-wise operation can we perform on the register holding the 32-bit pattern to set all bits in the register to zero except the least signficant 2 bits? Write this as a single line of code.

To keep only the least significant 2 bits of a 32-bit value in a register (for example register R0), we can use a bitwise AND operation with a mask that has the least significant 2 bits set to 1 and all other bits set to 0. The mask for this operation is ‘0b00000000000000000000000000000011’, which is 3 decimals.

Question 8.3.1(b): Using a label named "select:". Write the code needed to repeatedly sample a random number (from. Random) until the value is in the range 1-3.  For now, just write this as a seperate program and test it.

Question 8.3.2(a): Write the ARM assembly code that implements the algorithm expressed in the psuedo code above. Implement this as a seperate stand alone program and initialise R0 with a number at the start of your program to allow you to test the functionality.  You wil want to test it using different values in R0.

Part 8.4: Implementing the full game